Contents

1	Instruction to this template	5

1.1	Document context	5

1.2	Document metadata	5

2	Version History	6

3	Purpose of the document	7

3.1	Background	7

3.2	Scope and Audience	7

3.3	Including/excluding	7

3.4	Disclaimer	7

3.5	Versioning and Traceability	7

4	Service Overview	8

4.1	2 Steps approach:	8

4.1.1	Step1, before SSB retune including neighboring sites	8

4.1.2	Step2, SSB retune cluster	8

4.1.3	Step2 Cleanup	8

4.2	Coexistence of old & new SSB:	8

4.3	Clusters definition and pre-checks	10

4.4	Tool & Service Roadmap readiness	12

5	Tool Overview	13

6	Tool Inputs	13

7	Tool Outputs	13

7.1	ConfigurationAudit outputs	13

7.2	ConsistencyChecks outputs	13

8	ConfigurationAudit Module	15

8.1	Execution Modes	15

8.2	What ConfigurationAudit Checks	15

9	ConsistencyChecks Module	16

9.1	Execution Modes	16

9.2	What ConsistencyChecks Detects	16

9.3	Missing Relations	16

9.4	New Relations	16

9.5	Parameters Discrepancies	16

9.6	Frequency Discrepancies (SSB)	16

10	Correction Commands	17

10.1	Managed Objects producing correction commands	17

10.2	Conditions (user-facing)	17

10.3	Command examples (as generated by the tool)	17

11	Appendix A – SummaryAudit Metrics (Detailed Specification)	19

11.1	Cardinality Audit	19

11.1.1	Cardinality GUtranFreqRelation Table	19

11.1.2	Cardinality GUtranSyncSignalFrequency Table	20

11.1.3	Cardinality NRFreqRelation Table	21

11.1.4	Cardinality NRFrequency Table	22

11.2	Cardinality Inconsistencies	24

11.2.1	Cardinality GUtranFreqRelation Table	24

11.2.2	Cardinality GUtranSyncSignalFrequency Table	24

11.2.3	Cardinality NRFreqRelation Table	24

11.2.4	Cardinality NRFrequency Table	25

11.3	ENDC Audit	25

11.3.1	EndcDistrProfile	25

11.3.2	FreqPrioNR	27

11.4	ENDC Inconsistencies	30

11.4.1	EndcDistrProfile	30

11.4.2	FreqPrioNR	30

11.5	EndcProfileAudit	31

11.5.1	FreqPrioNR	31

11.6	External / TermPoint Audit	31

11.6.1	TermPointToGNB	31

11.7	LTE Frequency Audit	33

11.7.1	GUtranSyncSignalFrequency	33

11.7.2	ExternalGUtranCell	33

11.7.3	GUtranCellRelation	36

11.7.4	GUtranFreqRelation	38

11.7.5	GUtranSyncSignalFrequency	42

11.8	LTE Frequency Inconsistencies	46

11.8.1	GUtranFreqRelation	46

11.8.2	GUtranSyncSignalFrequency	47

11.9	NR Frequency Audit	48

11.9.1	ExternalNRCellCU	48

11.9.2	NRCellDU	49

11.9.3	NRCellRelation	54

11.9.4	NRFreqRelation	56

11.9.5	NRFrequency	61

11.9.6	NRSectorCarrier	67

11.10	NR Frequency Inconsistencies	70

11.10.1	NRCellDU	70

11.10.2	NRFreqRelation	71

11.10.3	NRFrequency	73

11.10.4	NRSectorCarrier	74

11.11	NR Termpoint Audit	75

11.11.1	TermPointToGNodeB	75

11.12	X2 Termpoint Audit	77

11.12.1	TermPointToENodeB	77

11.12.2	TermPointToGNB	78

# Instruction to this template

This chapter is an instruction for the user of this template on which the live document “Sys Doc - User Guide” is based. When the live document is created, chapter 1 shall be deleted.

## Document context

The document describes involved parties of the IT system.

## Document metadata

This document is a template for one of several document types in order to describe an IT system in the Ericsson environment, referred to as the System documentation.

This template must be changed to a live document by making the following changes in the document header (use the Word tool Update Ericsson Header).

Document Title: Sys Doc - User Guide

Document Name (type of document): USER GUIDE

Document No (decimal class): 198 17-<product number> Uen 
(do this after the document is imported into EriDoc using Assign Document Number function).

# Version History

| Revision | Drafted By | Date | Approved By | Summary |
| --- | --- | --- | --- | --- |
| PA0 | Jaime Tur (EJAITUR) | 01/02/2026 |  | Initial draft |
| PA1 | Jaime Tur (EJAITUR) | 01/02/2026 |  | Added version history. Removed Examples section referring to old documents |

# Purpose of the document

This document is intended for delivery to third parties. It describes required inputs, execution modes, outputs, all SummaryAudit metrics (with code-based trigger conditions), ConsistencyChecks behavior, and correction command generation.

## Background

This document provides the official user-facing documentation for the SSB Retuning Automations tool. It explains how to prepare inputs, how each module behaves, which execution modes are available, and how to interpret all generated outputs.

## Scope and Audience

Audience: telecom engineers and operational teams. The content is user-facing: no source-code knowledge is required. Metric titles are reproduced exactly as generated by the tool.

## Including/excluding

<Note what is included in the user guide and not included.

## Disclaimer

This tool and documentation are provided as-is. Always validate results before applying any changes to live networks. Correction commands must be reviewed and approved according to your operational process.

## Versioning and Traceability

This tool and documentation are provided as-is. Always validate results before applying any changes to live networks. Correction commands must be reviewed and approved according to your operational process.

# Service Overview

SSB Retuning Automations parses network logs, performs ConfigurationAudit checks, optionally compares PRE/POST states with ConsistencyChecks, and generates Excel deliverables and correction command exports.

This document provides the official user-facing documentation for the SSB Retuning Automations tool. It explains how to prepare inputs, how each module behaves, which execution modes are available, and how to interpret all generated outputs.

## 2 Steps approach:

### Step1, before SSB retune including neighboring sites

New GUtranSyncSignalFrequency/GUtranFreqRelation and NRFrequency/NRFreqRelation instances following VZ naming convention will be added manually before SSB retune

Low PRIO for new SSB: endcB1MeasPriority 2->0, UeMCNrFreqRelProfile=Prio5->Prio4

Add new SSB to FreqPrioNR and EndcDistrProfile.MandatoryGUtranFreqRef

### Step2, SSB retune cluster

SSB retune

Low PRIO for old SSB: endcB1MeasPriority 2->0, UeMCNrFreqRelProfile=Prio5->Prio4

Normal PRIO for new SSB: endcB1MeasPriority 2, UeMCNrFreqRelProfile=Prio5

Replace old SSB on EndcDistrProfile gUtranFreqRef

### Step2 Cleanup

Update SSB in Relations that failed to update automatically (unavailableTermpoints)

Remove CellRelations that ANR might have added due to outage during SSB retune

## Coexistence of old & new SSB:

Coexistence of old/new SSB will split UE measurements in 2 frequencies. A cell ranked 3rd might become 1st after the split, increasing the risk of connecting to overshooting cells.

To handle this, suggest setting lower priority for the SSB not collocated to minimize potential issues due to overshooting

Retune cluster sites with low priority to old SSB (priority change on Step2)

Neighboring sites with low priority to new SSB

UEs camping on neighboring sites to retune cluster will not try to reselect or add ENDC on new SSB (lower priority)

UEs camping on border sites in retune cluster will not try to reselect or add ENDC on old SSB (lower priority)

Step1 will add FreqRelation to new SSB in retune + neighboring clusters

Step2 will not remove FreqRelation to old SSB

ENDC and Mobility allowed in border and neighboring sites with existing CellRelations

## Clusters definition and pre-checks

For each ENM identify retune+border clusters with markets being planned on different nights

Step0 all markets: Pre-checks export with all relevant MOs and params in retune+border clusters, including cellRelation. Evaluate the number of MOs and anticipate any additions that might be needed to the scripts for different markets

Step1 all clusters and neighboring markets to create all definitions needed for Old & new SSB  co-existence:

  - New GUtranSyncSignalFrequency/GUtranFreqRelation and NRFrequency/NRFreqRelation instances following VZ naming convention will be added manually before SSB retune

  - Mobility/Anchoring lower PRIO for new SSB: endcB1MeasPriority 0, cellReselectionSubPriority 0  (TBC)

  - Add new SSB to FreqPrioNR and EndcDistrProfile.MandatoryGUtranFreqRef

Step2, cluster of 1 or more markets each night during maintenance window

  - SSB retune

  - Mobility/Anchoring lower PRIO for old SSB (TBC endcB1MeasPriority 0, cellReselectionSubPriority 0) , normal priority for new SSB

  - Update EndcDistrProfile replacing old SSB in gUtranFreqRef

  - Change MCPC profiles containing old SSB names to new SSB names

Post Step2 full audit (including cellRelation)

  - Additional cellRelation instances might be created during night works, due to outages during SSB retune.

  - Full audit and comparison of CellRelation to detect any additions and flag them for deletion

Step2 Cleanup:

  - Remove CellRelations that ANR might have added during SSB retune cells locked

  - Change MCPC profiles containing old SSB names to new SSB names

  - Final cleanup after all clusters done in one ENM and no borders with old SSB (including other vendor areas)

  - There should not be any CellRelation pointing to old SSB Freq

  - Remove old SSB GUtranSyncSignalFrequency/GUtranFreqRelation and NRFrequency/NRFreqRelation

  - Remove MCPC profiles containing old SSB names, not referenced anymore after NRFreqRelation deletion

## Tool & Service Roadmap readiness

Current roadmap status can be shown in the following picture

# Tool Overview

SSB Retuning Automations parses network logs, performs ConfigurationAudit checks, optionally compares PRE/POST states with ConsistencyChecks, and generates Excel deliverables and correction command exports.

The tool is composed of 2 main Modules:

Configuration Audit module

Consistency Check module

# Tool Inputs

Inputs are folders (or ZIP archives) containing log files. The tool parses the logs into MO tables and runs checks on them. For ConsistencyChecks, provide either explicit PRE and POST folders or a single root folder where PRE/POST runs can be auto-detected.

[Screenshot placeholder – Example input folder structure]

# Tool Outputs

Each module generates a dedicated output folder containing Excel reports, logs, and optional correction command exports.

## ConfigurationAudit outputs

ConfigurationAudit produces an output folder containing:

An Excel audit workbook with a SummaryAudit sheet and multiple MO sheets.

Optional correction command exports (folder/ZIP) when enabled.

Execution logs.

## ConsistencyChecks outputs

ConsistencyChecks produces an output folder containing:

An Excel workbook summarizing PRE/POST comparison results.

Tables for Missing Relations, New Relations, and Discrepancies (NR and GU).

Optional correction command exports for relations.

Execution logs.

# ConfigurationAudit Module

ConfigurationAudit performs a detailed validation of a single configuration snapshot. It parses network logs, builds Managed Object (MO) tables, and evaluates multiple consistency and retuning-related rules.

## Execution Modes

ConfigurationAudit can be executed in two main modes:

- Normal mode: The user provides a single input folder. The tool runs all checks and generates one audit output.

- Batch mode: The user provides a root folder containing multiple Step0 runs. The tool automatically detects valid runs and executes ConfigurationAudit for each one.

## What ConfigurationAudit Checks

ConfigurationAudit validates frequency configuration (SSB/ARFCN), relations, externals, TermPoints, profiles, and cardinality constraints. Results are consolidated in the SummaryAudit sheet.

# ConsistencyChecks Module

ConsistencyChecks compares PRE and POST relation tables (NRCellRelation and GUtranCellRelation). The comparison uses a stable key per relation (e.g., NodeId + CellId + RelationId).

## Execution Modes

- Normal (manual PRE/POST): The user explicitly provides PRE and POST input folders.

- Batch (auto-detected PRE/POST):The user provides a root folder. The tool automatically selects PRE and POST runs based on timestamps and naming rules.

## What ConsistencyChecks Detects

The module detects missing relations, new relations, and discrepancies (parameter differences and frequency differences). Results are summarized in the comparison Excel.

## Missing Relations

A relation is classified as MISSING in POST when its key exists in PRE but does not exist in POST. In code terms: Missing = PRE_keys − POST_keys.

## New Relations

A relation is classified as NEW in POST when its key exists in POST but does not exist in PRE. In code terms: New = POST_keys − PRE_keys.

## Parameters Discrepancies

A relation is classified as a DISCREPANCY when the key exists in both PRE and POST, but one or more audited parameters differ. The tool compares shared columns, excluding technical columns (Pre/Post, Date), key columns, and the detected frequency column.

## Frequency Discrepancies (SSB)

Frequency discrepancies are identified by extracting a normalized base frequency (Freq_Pre / Freq_Post) and applying retuning rules. Typical rule: if a relation has the target frequency in PRE (or already has POST) but POST does not contain the expected POST frequency, it is flagged for frequency discrepancy handling

# Correction Commands

Correction commands are generated by building a 'Correction_Cmd' column for specific sheets. Commands must be reviewed before execution.

## Managed Objects producing correction commands

The tool generates correction commands for at least the following MO families (based on the correction command builder and exporters):

- NRCellRelation (new/missing/discrepancies)

- GUtranCellRelation (new/missing/discrepancies)

- ExternalNRCellCU

- ExternalGUtranCell

- TermPointToGNodeB / TermPointToGNB

Additionally, ConfigurationAudit can export commands for any sheet containing a 'Correction_Cmd' column.

## Conditions (user-facing)

- New/Missing relations:

  - NEW: relation key exists only in POST.

  - MISSING: relation key exists only in PRE.

- Discrepancies:

  - Relation key exists in both sides but audited parameters differ.

- External/TermPoint commands:

  - Generated when the external/termpoint reference does not match the expected target type or expected retuning-side SSB/profile behavior.

## Command examples (as generated by the tool)

Examples below illustrate the typical command format. Exact values are filled from the corresponding MO row.

“set EUtranCellFDD={eu_cell},GUtranFreqRelation={freq_rel},GUtranCellRelation={cell_rel} coverageIndicator {coverage}"

“set {clean_nrcell_ref_short} nRFrequencyRef NRNetwork=1,NRFrequency={n77_ssb_post}-30" if clean_nrcell_ref_short else "",

“set NRCellCU={nr_cell_cu},NRCellRelation={nr_cell_rel} coverageIndicator {coverage}"

“set NRCellCU={nr_cell_cu},NRCellRelation={nr_cell_rel} sCellCandidate {s_cell_candidate}"

“set ExternalGNBCUCPFunction={ext_gnb},ExternalNRCellCU={ext_cell} nRFrequencyRef {nr_tail}\n"

# Appendix A – SummaryAudit Metrics (Detailed Specification)

This appendix lists and explains every SummaryAudit metric generated by the tool. Metric titles are reproduced exactly as they appear in the Excel SummaryAudit sheet. For each metric, the trigger conditions are derived from the implementation logic in the processing modules.

## Cardinality Audit

### Cardinality GUtranFreqRelation Table

#### Max GUtranFreqRelation per LTE cell (limit 16)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranFreqRelation per LTE cell (required cell column missing)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranFreqRelation per LTE cell

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking GUtranFreqRelation cardinality

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### Cardinality GUtranSyncSignalFrequency Table

#### Max GUtranSyncSignalFrequency definitions per node (limit 24)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranSyncSignalFrequency per node (NodeId missing)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranSyncSignalFrequency per node

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking GUtranSyncSignalFrequency cardinality

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### Cardinality NRFreqRelation Table

#### Max NRFreqRelation per NR cell (limit 16)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFreqRelation per cell (required cell column missing)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFreqRelation per cell

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking NRFreqRelation cardinality

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### Cardinality NRFrequency Table

#### Max NRFrequency definitions per node (limit 64)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFrequency per node (NodeId missing)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFrequency per node

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking NRFrequency cardinality

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## Cardinality Inconsistencies

### Cardinality GUtranFreqRelation Table

#### LTE cells with GUtranFreqRelation count at limit 16

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### Cardinality GUtranSyncSignalFrequency Table

#### LTE nodes with GUtranSyncSignalFrequency definitions at limit 24

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### Cardinality NRFreqRelation Table

#### NR cells with NRFreqRelation count at limit 16

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### Cardinality NRFrequency Table

#### NR nodes with NRFrequency definitions at limit 64

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## ENDC Audit

### EndcDistrProfile

#### “Nodes with gUtranFreqRef containing the old N77 SSB ({n77_ssb_pre}) and the N77B SSB ({n77b_ssb}) (from EndcDistrProfile table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “Nodes with gUtranFreqRef containing the new N77 SSB ({n77_ssb_post}) and the N77B SSB ({n77b_ssb}) (from EndcDistrProfile table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### EndcDistrProfile table present but NodeId/gUtranFreqRef missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### EndcDistrProfile table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking EndcDistrProfile gUtranFreqRef

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### FreqPrioNR

#### “LTE nodes with the old N77 SSB ({old_ssb}) but without the new N77 SSB ({new_ssb}) (from FreqPrioNR table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with both, the old N77 SSB ({old_ssb}) and the new N77 SSB ({new_ssb}) (from FreqPrioNR table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR nodes with RATFreqPrioId = 'fwa' in N77 band (from FreqPrioNR table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR nodes with RATFreqPrioId = 'publicsafety' in N77 band (from FreqPrioNR table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### FreqPrioNR table has no N77 rows (based on FreqPrioNRId)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### FreqPrioNR table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking FreqPrioNR

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## ENDC Inconsistencies

### EndcDistrProfile

#### “Nodes with gUtranFreqRef not containing N77 SSBs ({n77_ssb_pre} or {n77_ssb_post}) together with N77B SSB ({n77b_ssb}) (from EndcDistrProfile table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### FreqPrioNR

#### “LTE cells with mismatching params between FreqPrioNR {old_ssb} and {new_ssb}"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR nodes with RATFreqPrioId different from 'fwa'/'publicsafety' in N77 band (from FreqPrioNR table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## EndcProfileAudit

### FreqPrioNR

#### FreqPrioNR table present but NodeId/FreqPrioNRId/RATFreqPrioId missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## External / TermPoint Audit

### TermPointToGNB

#### Error while checking TermPointToGNB

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## LTE Frequency Audit

### GUtranSyncSignalFrequency

#### "LTE LowMidBand Nodes (from GUtranSyncSignalFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

mmWave classification: SSB in [2,000,000 … 2,100,000] is treated as mmWave; otherwise LowMidBand.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### "LTE mmWave Nodes (from GUtranSyncSignalFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### ExternalGUtranCell

#### “External cells to old N77 SSB ({old_ssb}) (from ExternalGUtranCell)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “External cells to new N77 SSB ({new_ssb}) (from ExternalGUtranCell)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “External cells to old N77 SSB ({old_ssb}) with serviceStatus=OUT_OF_SERVICE (from ExternalGUtranCell)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “External cells to new N77 SSB ({new_ssb}) with serviceStatus=OUT_OF_SERVICE (from ExternalGUtranCell)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### External cells OUT_OF_SERVICE checks skipped (serviceStatus missing)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### ExternalGUtranCell table present but NodeId / gUtranSyncSignalFrequencyRef column missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### ExternalGUtranCell table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking ExternalGUtranCell

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### GUtranCellRelation

#### “LTE cellRelations to old N77 SSB ({old_ssb}) (from GUtranCellRelation table)"

What this means: This metric reports how many relations match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE cellRelations to new N77 SSB ({new_ssb}) (from GUtranCellRelation table)"

What this means: This metric reports how many relations match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranCellRelation table present but NodeId / GUtranFreqRelationId column missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranCellRelation table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking GUtranCellRelation

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### GUtranFreqRelation

#### “LTE nodes with the old N77 SSB ({n77_ssb_pre}) (from GUtranFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with the new N77 SSB ({n77_ssb_post}) (from GUtranFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with both, the old N77 SSB ({n77_ssb_pre}) and the new N77 SSB ({n77_ssb_post}) (from GUtranFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with the old N77 SSB ({n77_ssb_pre}) but without the new SSB ({n77_ssb_post}) (from GUtranFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE cells with GUtranFreqRelationId {expected_old_rel_id} and {expected_new_rel_id} (from GUtranFreqRelation table)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE cells with GUtranFreqRelationId {expected_old_rel_id} but without {expected_new_rel_id} (from GUtranFreqRelation table)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranFreqRelation cell-level check skipped (EUtranCellFDDId/EUtranCellId/CellId/GUCellId missing)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranFreqRelation table present but SSB/NodeId missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranFreqRelation table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking GUtranFreqRelation

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### GUtranSyncSignalFrequency

#### LTE nodes with GUtranSyncSignalFrequency defined (from GUtranSyncSignalFrequency table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

mmWave classification: SSB in [2,000,000 … 2,100,000] is treated as mmWave; otherwise LowMidBand.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with the old N77 SSB ({n77_ssb_pre}) (from GUtranSyncSignalFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with the new N77 SSB ({n77_ssb_post}) (from GUtranSyncSignalFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with both, the old N77 SSB ({n77_ssb_pre}) and the new N77 SSB ({n77_ssb_post}) (from GUtranSyncSignalFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with the old N77 SSB ({n77_ssb_pre}) but without the new N77 SSB ({n77_ssb_post}) (from GUtranSyncSignalFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranSyncSignalFrequency table present but required columns missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Triggered when the referenced MO table is missing/empty or the required columns are not present.

How to interpret/act: Fix the input dataset (missing logs / missing MO) and re-run; otherwise this area of the audit is incomplete.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### GUtranSyncSignalFrequency table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking GUtranSyncSignalFrequency

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## LTE Frequency Inconsistencies

### GUtranFreqRelation

#### “LTE nodes with the N77 SSB not in ({n77_ssb_pre}, {n77_ssb_post}) (from GUtranFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE nodes with Auto-created GUtranFreqRelationId to new N77 SSB ({n77_ssb_post}) but not following VZ naming convention ({n77_ssb_post}-30-20-0-1) (from GUtranFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “LTE cells with mismatching params between GUtranFreqRelationId {expected_old_rel_id} and {expected_new_rel_id} (from GUtranFreqRelation table)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### GUtranSyncSignalFrequency

#### “LTE nodes with the N77 SSB not in ({n77_ssb_pre}, {n77_ssb_post}) (from GUtranSyncSignalFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## NR Frequency Audit

### ExternalNRCellCU

#### “External cells to old N77 SSB ({old_ssb}) (from ExternalNRCellCU)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “External cells to new N77 SSB ({new_ssb}) (from ExternalNRCellCU)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### ExternalNRCellCU table present but NodeId / nRFrequencyRef missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### ExternalNRCellCU table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking ExternalNRCellCU

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### NRCellDU

#### NR Nodes with ssbFrequency (from NRCellDU table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

mmWave classification: SSB in [2,000,000 … 2,100,000] is treated as mmWave; otherwise LowMidBand.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR LowMidBand Nodes (from NRCellDU table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

mmWave classification: SSB in [2,000,000 … 2,100,000] is treated as mmWave; otherwise LowMidBand.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR mmWave Nodes (from NRCellDU table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

mmWave classification: SSB in [2,000,000 … 2,100,000] is treated as mmWave; otherwise LowMidBand.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR Nodes with both LowMidBand and mmWave NR cells (from NRCellDU table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

mmWave classification: SSB in [2,000,000 … 2,100,000] is treated as mmWave; otherwise LowMidBand.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR nodes with N77 SSB in band (646600-660000) (from NRCellDU table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with N77 SSB in Pre-Retune allowed list ({allowed_pre_str}) (from NRCellDU table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

For “allowed list” metrics, the node must have at least one valid N77 SSB and ALL its valid N77 SSB values must belong to the configured allowed list.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with N77 SSB in Post-Retune allowed list ({allowed_post_str}) (from NRCellDU table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

For “allowed list” metrics, the node must have at least one valid N77 SSB and ALL its valid N77 SSB values must belong to the configured allowed list.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRCellDU table has no N77 rows

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “allowed list” metrics, the node must have at least one valid N77 SSB and ALL its valid N77 SSB values must belong to the configured allowed list.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRCellDU table present but required columns missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “allowed list” metrics, the node must have at least one valid N77 SSB and ALL its valid N77 SSB values must belong to the configured allowed list.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

Triggered when the referenced MO table is missing/empty or the required columns are not present.

How to interpret/act: Fix the input dataset (missing logs / missing MO) and re-run; otherwise this area of the audit is incomplete.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRCellDU table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “allowed list” metrics, the node must have at least one valid N77 SSB and ALL its valid N77 SSB values must belong to the configured allowed list.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking NRCellDU

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “allowed list” metrics, the node must have at least one valid N77 SSB and ALL its valid N77 SSB values must belong to the configured allowed list.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### NRCellRelation

#### “NR cellRelations to old N77 SSB ({old_ssb}) (from NRCellRelation table)"

What this means: This metric reports how many relations match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR cellRelations to new N77 SSB ({new_ssb}) (from NRCellRelation table)"

What this means: This metric reports how many relations match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRCellRelation table present but NodeId / nRFreqRelationRef column missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRCellRelation table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking NRCellRelation

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### NRFreqRelation

#### “NR nodes with the old N77 SSB ({n77_ssb_pre}) (from NRFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with the new N77 SSB ({n77_ssb_post}) (from NRFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with both, the old N77 SSB ({n77_ssb_pre}) and the new N77 SSB ({n77_ssb_post}) (from NRFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with the old N77 SSB ({n77_ssb_pre}) but without the new N77 SSB ({n77_ssb_post}) (from NRFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR cells with the old N77 SSB ({n77_ssb_pre}) and the new SSB ({n77_ssb_post}) (from NRFreqRelation table)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR cells with the old N77 SSB ({n77_ssb_pre}) but without new N77 SSB ({n77_ssb_post}) (from NRFreqRelation table)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with the old N77 SSB ({n77_ssb_pre}) and the new SSB ({n77_ssb_post}) NRFreqRelation pointing to same mcpcPCellNrFreqRelProfileRef containing old SSB name (from NRFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with the new N77 SSB ({n77_ssb_post}) NRFreqRelation pointing to mcpcPCellNrFreqRelProfileRef containing new SSB name (cloned) or Other (from NRFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFreqRelation cell-level check skipped (NRCellCUId/NRCellId/CellId missing)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFreqRelation table has no N77 rows

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFreqRelation table present but SSB/NodeId missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFreqRelation table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking NRFreqRelation

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### NRFrequency

#### “NR nodes with N77 SSB defined (from NRFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with the old N77 SSB ({n77_ssb_pre}) (from NRFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with the new N77 SSB ({n77_ssb_post}) (from NRFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with both, the old N77 SSB ({n77_ssb_pre}) and the new N77 SSB ({n77_ssb_post}) (from NRFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with the old N77 SSB ({n77_ssb_pre}) but without the new N77 SSB ({n77_ssb_post}) (from NRFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFrequency table has no N77 rows

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFrequency table present but required columns missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Triggered when the referenced MO table is missing/empty or the required columns are not present.

How to interpret/act: Fix the input dataset (missing logs / missing MO) and re-run; otherwise this area of the audit is incomplete.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFrequency table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking NRFrequency

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### NRSectorCarrier

#### NR nodes with N77 ARCFN in band (646600-660000) (from NRSectorCarrier table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with N77 ARCFN in Pre-Retune allowed list ({allowed_pre_str}) (from NRSectorCarrier table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with N77 ARCFN in Post-Retune allowed list ({allowed_post_str}) (from NRSectorCarrier table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRSectorCarrier table present but required columns missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

Triggered when the referenced MO table is missing/empty or the required columns are not present.

How to interpret/act: Fix the input dataset (missing logs / missing MO) and re-run; otherwise this area of the audit is incomplete.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRSectorCarrier table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking NRSectorCarrier

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## NR Frequency Inconsistencies

### NRCellDU

#### NR nodes with N77 SSB not in Pre/Post Retune allowed lists (from NRCellDU table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “allowed list” metrics, the node must have at least one valid N77 SSB and ALL its valid N77 SSB values must belong to the configured allowed list.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR nodes with N77 SSB not in Pre/Post Retune allowed lists (no pre/post allowed lists configured) (from NRCellDU table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “allowed list” metrics, the node must have at least one valid N77 SSB and ALL its valid N77 SSB values must belong to the configured allowed list.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### NRFreqRelation

#### “NR nodes with the N77 SSB not in ({n77_ssb_pre}, {n77_ssb_post}) (from NRFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR nodes with Auto-created NRFreqRelationId to new N77 SSB ({n77_ssb_post}) but not following VZ naming convention (e.g. with extra characters: 'auto_{n77_ssb_post}')"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR Nodes with the new N77 SSB ({n77_ssb_post}) and NRFreqRelation reference to McpcPCellNrFreqRelProfile with old SSB before '_' ({n77_ssb_pre}_xxxx) (from NRFreqRelation table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NRFreqRelation mcpcPCellNrFreqRelProfileRef clone check skipped (mcpcPCellNrFreqRelProfileRef missing)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### “NR cells with mismatching params between old N77 SSB ({n77_ssb_pre}) and the new N77 SSB ({n77_ssb_post}) (from NRFreqRelation table)"

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### NRFrequency

#### “NR nodes with the N77 SSB not in ({n77_ssb_pre}, {n77_ssb_post}) (from NRFrequency table)"

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

A row is treated as N77 when the frequency is within 646600–660000 (N77 band check).

A node is counted if ANY of its N77 rows contains the expected PRE (old) SSB/ARFCN value.

A node is counted if ANY of its N77 rows contains the expected POST (new) SSB/ARFCN value.

“Both old and new” is computed as the intersection of nodes having old value and nodes having new value.

“Old without new” is computed as nodes having the old value minus nodes having the new value.

“Not in (old, new)” is triggered when a node has N77 rows but none of the valid values match either expected PRE or POST value.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### NRSectorCarrier

#### NR nodes with N77 ARCFN not in Pre/Post Retune allowed lists (from NRSectorCarrier table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR nodes with N77 ARCFN not in Pre/Post Retune allowed lists (no pre/post allowed lists configured) (from NRSectorCarrier table)

What this means: This metric reports how many unique nodes match the condition described in the metric title.

Trigger conditions (from code):

Frequency values are parsed to integer; non-parsable values are ignored for numeric checks.

For “not in allowed lists” metrics, at least one valid N77 SSB value must be outside the union of PRE and POST allowed lists.

Results are aggregated by NodeId and reported as unique node counts.

How to interpret/act: If Value > 0, treat this as an inconsistency: validate frequencies/refs and consider correction commands or re-run of the relevant procedure.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## NR Termpoint Audit

### TermPointToGNodeB

#### TermPointToGNodeB table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### TermPointToGNodeB table present but required columns missing

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Triggered when the referenced MO table is missing/empty or the required columns are not present.

How to interpret/act: Fix the input dataset (missing logs / missing MO) and re-run; otherwise this area of the audit is incomplete.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR to NR TermPoints with administrativeState=LOCKED (from TermPointToGNodeB)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR to NR TermPoints with operationalState=DISABLED (from TermPointToGNodeB)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking TermPointToGNodeB

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

## X2 Termpoint Audit

### TermPointToENodeB

#### NR to LTE TermPoints with administrativeState=LOCKED (from TermPointToENodeB)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### NR to LTE TermPoints with operationalState=DISABLED (from TermPointToENodeB)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### TermPointToENodeB table present but required columns missing (NodeId/ExternalENodeBFunctionId/admin/oper)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Triggered when the referenced MO table is missing/empty or the required columns are not present.

How to interpret/act: Fix the input dataset (missing logs / missing MO) and re-run; otherwise this area of the audit is incomplete.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### TermPointToENodeB table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking TermPointToENodeB

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

### TermPointToGNB

#### TermPointToGNB table present but required columns missing (NodeId/ExternalGNBCUCPFunctionId/admin/oper/ip)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Required columns are detected case-insensitively.

Triggered when the referenced MO table is missing/empty or the required columns are not present.

How to interpret/act: Fix the input dataset (missing logs / missing MO) and re-run; otherwise this area of the audit is incomplete.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### LTE to NR TermPoints with administrativeState=LOCKED (from TermPointToGNB)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### LTE to NR TermPoints with operationalState=DISABLED (from TermPointToGNB)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### LTE to NR TermPoints with usedIpAddress=0.0.0.0/:: (from TermPointToGNB)

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### TermPointToGNB table

What this means: This metric reports the result of the check described in the metric title.

Trigger conditions (from code):

Condition follows the wording of the metric title; see the referenced table/MO and ExtraInfo for affected objects.

How to interpret/act: If Value > 0, review the affected objects listed in ExtraInfo and validate the configuration in the corresponding MO table.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.

#### Error while checking TermPointToGNB

What this means: This metric indicates that an error occurred while executing the check.

Trigger conditions (from code):

Triggered when the tool raises an exception during this check; the exception text is reported as the Value.

How to interpret/act: Check the execution logs, fix the underlying cause (parsing/input), and re-run the audit.

Primary source: SummaryAudit → ExtraInfo lists affected objects when available.