Contents

1	Heading 1	4

Document Title: SSB Retuning Automations - User Guide

Document Name (type of document): USER GUIDE

Document No (decimal class): 198 17-<product number> Uen 
(do this after the document is imported into EriDoc using Assign Document Number function).

# Heading 1

# Technical User Guide — SSB Retuning Automations

# Tool overview

SSB Retuning Automations is an automation platform for SSB retuning projects that can run in GUI or CLI mode or through a Web Interface (using a server/client infrastructure) and orchestrates five functional modules:

Module 0: Update Network Frequencies.

Module 1: Configuration Audit & Logs Parser.

Module 2: Consistency Check (manual Pre/Post).

Module 3: Consistency Check Bulk (automatic Pre/Post detection by market).

Module 4: Final Clean-Up.

The main execution lives in "src/SSB_RetuningAutomations.py", where CLI arguments, GUI, configuration persistence, input resolution (folders/ZIP), per-module execution, and artifact versioning are managed.

# Repository technical architecture

## Main files

### Orchestration core

"src/SSB_RetuningAutomations.py": entry point, CLI/GUI parsing, module routing, batch/bulk execution, and versioning.

### Main modules files

"src/modules/ConfigurationAudit/ConfigurationAudit.py": log parsing and audit workbook construction (Excel + PPT).

"src/modules/ConfigurationAudit/ca_summary_excel.py": assembly of "SummaryAudit" and discrepancy dataframes.

"src/modules/ConsistencyChecks/ConsistencyChecks.py": PRE/POST loading, relation comparison, discrepancies, and output export.

"src/modules/ProfilesAudit/ProfilesAudit.py": profiles audit (integrated into module 1).

"src/modules/CleanUp/FinalCleanUp.py": final clean-up (base implementation for extension).

### Common layer and utilities

"src/modules/Common/*.py": correction command logic and shared functions.

"src/utils/*.py": IO, parsing, frequency handling, Excel, pivots, sorting, infrastructure, and timing.

# Inputs, outputs, and content per module

## Module 0 — Update Network Frequencies

### Input

Input folder (may contain subfolders/ZIPs already supported by the IO layer).

Logs with an "NRFrequency" table and the "arfcnValueNRDl" column.

### Process

1. Scan logs and detects "NRFrequency" blocks.

2. Extracts numeric values from "arfcnValueNRDl".

3. Removes duplicates and sorts frequencies.

4. Updates the persisted “Network frequencies” configuration for GUI/CLI.

### Output

Does not generate Excel/PPT.

Updates the persisted network frequency value used for filtering and selection in later runs.

## Module 1 — Configuration Audit & Logs Parser

### Inputs

Input folder with logs (".log", ".logs", ".txt") or ZIPs resolvable by utilities.

Frequency parameters:

"n77_ssb_pre"

"n77_ssb_post"

"n77b_ssb"

allowed SSB/ARFCN lists pre/post.

Flags:

"profiles_audit"

"frequency_audit"

"export_correction_cmd"

"fast_excel_export".

### Process

1. Parses files and extracts MO tables by "SubNetwork" blocks.

2. Generates one sheet per detected table.

3. Builds "SummaryAudit" + pivots/auxiliary summaries.

4. Runs profiles audit if enabled.

5. Exports CA correction commands if requested.

6. Generates the summary PPT.

### Outputs

Folder "ConfigurationAudit_<timestamp>_v<version>/".

Excel file "ConfigurationAudit_<timestamp>_v<version>.xlsx":

Sheets for each parsed MO table.

"SummaryAudit".

NR/LTE parameter discrepancy sheets.

Summary/pivot sheets by frequencies and relations.

PPT file "ConfigurationAudit_<timestamp>_v<version>.pptx".

Optional folder "Correction_Cmd_CA/" with AMOS commands.

### Main semantic content

SummaryAudit contains rows with:

"Category", "SubCategory", "Metric", "Value", "ExtraInfo",

and execution context fields (stage, module, etc. depending on the flow).

"Value" usually represents a count of impacted nodes/cells/relations.

"ExtraInfo" contains the NodeId list or a compact discrepancy detail.

## Module 2 — Consistency Check (Pre/Post)

### Inputs

"input_pre" and "input_post" (or equivalent resolved structure).

Frequencies "n77_ssb_pre" and "n77_ssb_post".

Optional reference to PRE and POST "ConfigurationAudit" to enrich target classification.

Optional list of frequency filters ("cc_freq_filters").

### Process

1. Loads relation tables ("GUtranCellRelation", "NRCellRelation").

2. Normalizes columns/keys and selects the most recent snapshots by date.

3. Detect:

new relations,

missing relations,

parameter discrepancies,

frequency discrepancies,

summary by PRE/POST frequency pair.

4. Classify destination targets as "SSB-Pre", "SSB-Post" or "Unknown".

5. Exports detailed Excel outputs and correction commands.

### Outputs

"CellRelation_<timestamp>_v<version>.xlsx" (end-to-end relations view).

"ConsistencyChecks_CellRelation_<timestamp>_v<version>.xlsx" with:

"Summary"

"SummaryAuditComparisson" (if there is PRE/POST SummaryAudit)

"Summary_CellRelation"

GU blocks: "GU_relations", "GU_param_disc", "GU_freq_disc", "GU_freq_disc_unknown", "GU_missing", "GU_new"

NR blocks: "NR_relations", "NR_param_disc", "NR_freq_disc", "NR_freq_disc_unknown", "NR_missing", "NR_new"

optional "GU_all", "NR_all".

"Correction_Cmd_CC/" with commands per type (new/missing/discrepancies).

## Module 3 — Consistency Check Bulk

### Inputs

Root folder with subfolders like "yyyymmdd_hhmm_step0" (optionally nested by market).

### Process

1. Detects PRE/POST candidates by the most appropriate date/time.

2. Excludes folders using a blacklist ("ignore", "old", "bad", "partial", "incomplete", "discard", etc.).

3. Runs Module 2 for each detected market.

### Outputs

Same output structure as module 2, per market.

Traceability file "FoldersCompared.txt".

## Module 4 — Final Clean-Up

### Inputs

Final retune working folder.

### Process

Executes final cleanup policies (structure prepared to expand rules).

### Outputs

Versioned cleanup directory according to the active implementation.

# Configuration Audit module in detail

## SummaryAudit checks philosophy

SummaryAudit sheet contains a high-level checks table by categories. The flow:

1. Excludes "UNSYNCHRONIZED" nodes based on "MeContext".

2. Evaluates NR, LTE, ENDC, Externals, TermPoints, cardinalities, and profiles.

3. Records each check as a row ("Category/SubCategory/Metric/Value/ExtraInfo").

## Operational meaning of SummaryAudit rows

Category: audited technical domain (NR/LTE/ENDC/MeContext/etc.).

SubCategory: type of analysis (Audit/Inconsistencies/Profiles).

Metric: specific rule evaluated.

Value:

Integer: number of affected nodes/relations/cells.

"N/A": not evaluable due to missing columns.

Text: captured status or error.

ExtraInfo: list of nodes or bounded detail for troubleshooting.

## SummaryAudit checks catalog

### MeContext Audit

total unique nodes and unsynchronized node exclusion.

"UNSYNCHRONIZED" nodes (excluded from the rest of the audits).

### NR Frequency Audit / NR Frequency Inconsistencies

Source tables: "NRCellDU", "NRFrequency", "NRFreqRelation", "NRSectorCarrier", "NRCellRelation", "ExternalNRCellCU", "TermPointToGNodeB", "TermPointToGNB".

Main checks:

Detection of NR nodes with N77 SSB (band 646600–660000).

Classification of NR nodes as LowMidBand / mmWave / mixed.

Nodes whose N77 SSBs are fully within allowed PRE or POST lists.

Nodes with N77 SSB outside allowed lists.

Old/new SSB presence per node (only old, only new, both).

Nodes with NRFreqRelationId in an unexpected format (auto-created outside convention).

NR relations to old/new SSB.

NR externals and termpoints pointing to old/new/unknown.

Typical triggering:

Each check is enabled if the table and minimum required columns exist.

If columns are missing, a "N/A" status row is added.

If the table is empty or not found, an informative row "Table not found or empty" is added.

Interpretation:

"Value > 0" in inconsistencies indicates a real deviation that requires investigation.

"ExtraInfo" typically lists affected nodes for operational targeting.

### LTE Frequency Audit / LTE Frequency Inconsistencies

Source tables: "GUtranSyncSignalFrequency", "GUtranFreqRelation", "GUtranCellRelation", "ExternalGUtranCell", "TermPointToENodeB".

Main checks:

LTE nodes with old/new SSB.

Nodes with both old/new or old without new.

SSB outside the expected pre/post set.

LTE relations to old/new and parameter discrepancies per cell relation.

LTE externals OUT_OF_SERVICE for old/new.

### ENDC Audit / ENDC Inconsistencies

Source tables: "EndcDistrProfile", "FreqPrioNR".

Main checks:

"gUtranFreqRef" and "mandatoryGUtranFreqRef" with old/new + N77B combinations.

Nodes that do not contain the expected frequency combination.

In "FreqPrioNR": old without new, both present, and parameter mismatch per cell.

### Cardinalities Audit / Inconsistencies

Cardinality checks per relation table (per node and/or per cell) to detect overprovisioning or gaps versus expected limits.

### Profiles Audit (if enabled)

Compares PRE/POST profiles by supported profile MO.

Detects parameter discrepancies between old/new variants.

Adds results to SummaryAudit and auxiliary detail sheets.

# Consistency Check module in detail

## Filtering by non-retuned nodes

If a POST SummaryAudit exists, the module obtains PRE/POST node lists and can exclude discrepancies whose target points to nodes that did not complete retune, reducing operational noise.

## How it detects parameter discrepancies

1. Selects common PRE and POST relations by composite key:

GU: typically "NodeId", "EUtranCellFDDId", "GUtranCellRelationId".

NR: typically "NodeId", "NRCellCUId", "NRCellRelationId".

2. Excludes control columns (keys, frequency, Pre/Post, Date).

3. Compares value-by-value across shared columns.

4. Sets "ParamDiff=True" if at least one column differs.

5. In GU it ignores "timeOfCreation" and "mobilityStatusNR" to avoid false positives.

## How it detects frequency discrepancies

1. Extracts base frequency from relation references ("extract_gu_freq_base" / "extract_nr_freq_base").

2. Discrepancy rule:

if PRE had "freq_before" or "freq_after", and POST does not end up in "freq_after", it marks "FreqDiff=True".

3. Classifies the discrepancy as:

"FreqDiff_SSBPost" (target identified as SSB-Post),

"FreqDiff_Unknown" (cannot be associated to a known target).

## How it detects neighbor discrepancies

They are split into three groups:

New relations: keys present in POST and absent in PRE.

Missing relations: keys present in PRE and absent in POST.

Discrepancies: same key in PRE/POST but with parametric or frequency differences.

## Content of each ConsistencyChecks output sheet

Summary: KPIs per table (PRE/POST volume, discrepancies, new/missing, source files).

SummaryAuditComparisson: diff of SummaryAudit PRE vs POST metrics (without "ExtraInfo" to keep the comparison clean).

Summary_CellRelation: KPI per "Freq_Pre/Freq_Post" pair and per technology.

GU_relations / NR_relations: relation universe enriched with target classification and command snippets.

GU_param_disc / NR_param_disc: common relations with param differences.

GU_freq_disc / NR_freq_disc: frequency discrepancies to SSB-Post targets.

GU_freq_disc_unknown / NR_freq_disc_unknown: discrepancies with non-classifiable targets.

GU_missing / NR_missing: relations removed versus PRE.

GU_new / NR_new: relations added in POST.

GU_all / NR_all: optional consolidated dump for extended analysis.

# Input requirements and operational best practices

Keep market log exports in a consistent structure (especially for bulk).

Validate that PRE/POST have the same table granularity and consistent naming.

Validate frequency inputs ("n77_ssb_pre", "n77_ssb_post", "n77b_ssb") before batch execution.

Correctly configure allowed SSB/ARFCN lists to minimize false positives.

Run Configuration Audit before Consistency Checks whenever possible.

Use Bulk mode only with a controlled folder naming convention.

Review "Summary" and "Summary_CellRelation" first, then then deep-dive into detail sheets (ConfigurationAudit) and discrepancy tabs (ConsistencyCheck).

Consume "Correction_Cmd_CA" and "Correction_Cmd_CC" as a remediation proposal, not as blind execution.

# Execution Modes and Versioning

GUI mode: run without CLI arguments.

CLI mode: run with explicit module and options.

Web Interfacee: the tool can be run in a server/client infrastructure, accessing the server through a Web Interface where you can unpload your inputs, enqueue different tasks and  export the results when finish..

All Generated artifacts include a versioned suffix: "<timestamp>_v<TOOL_VERSION>".

This guarantees traceability and avoids collisions between runs.

# Known limitations and considerations

The engine depends on log quality and structure: missing columns downgrade checks to "N/A".

Some rules depend on naming conventions in references (NR/GU relation refs).

The Final Clean-Up module is prepared to extend operation-specific policies.

# Quick module reference

| Module | Main input | Main output | Goal |
| --- | --- | --- | --- |
| 0 Update Network Frequencies | Logs folder | Persisted config | Update network frequency list |
| 1 Configuration Audit | Logs/ZIP folder | Excel + PPT + CA commands | Audit configuration and profiles |
| 2 Consistency Check | PRE and POST folders | 2 Excel + CC commands | Compare pre/post relations |
| 3 Consistency Check (Bulk) | Multi-market root folder | Module 2 outputs per market | Run bulk comparison |
| 4 Final Clean-Up | Final folder | Clean-up folder | Operational final clean-up |